Notes to Users:

This document is a template.  The template was created by generalizing the Bridge Design Criteria report for a sizeable reconstruction project and therefore covers many bridge types and bridgework types.  Modify this document to fit each project.  This may mean deleting or adding sections to the template, changing design and/or material specifications, allowable values, materials, etc.  Please forward any suggestions for improving the template to the State Bridge Office.

nnn in the document indicates missing numerical data.

aaa in the document indicates missing alpha data.

Initial Date:		January 10, 1999

Modification Dates:	January 05, 2006

BRIDGE DESIGN CRITERIA (US)

U.S. nnn RECONSTRUCTION

KDOT Project No. nnn-nnn K-nnnn-01

aaaaaa County

DATE

Prepared by:  Your Company

TABLE OF CONTENTS

ITEM	PAGE

- Technical Policy Guidelines	n

- Geometric Layout	n

- Design Loads	n

- Materials	n

- Superstructure Design	n

- Substructure Design	n

- Miscellaneous	n

- Contract Pay Items	n

- Metric Conversion Factors	n

BRIDGE DESIGN CRITERIA (US)

aa nnn RECONSTRUCTION

Project No. nnn-nnn K-nnnn-01

1.	TECHNICAL POLICY GUIDELINES

The following design criteria identify the particular standards and procedures which are used for the bridge design:

1.1	2002 "AASHTO Standard Specifications for Highway Bridges", 17th Edition, plus   interims.

and/or

AASHTO LRFD Bridge Design Specifications, 3rd Edition plus interims.

1.2	KDOT "Bridge Design Manual" (January, 2005) and updates.

1.3	KDOT Standard Drawings (US)

1.4	State of Kansas "Standard Specifications for State Road and Bridge      Construction,"  1990 and Special Provisions.

  - Other publications as noted.

1.6	System of Weights and Measurements: The bridge design, plans, and specifi-cations are based upon the English (US) system of weights and measurements.

1.7 	CRSI Manual of Standard Practice

2.	GEOMETRIC LAYOUT

2.1	The bridge spans, horizontal and vertical alignment, and general arrangement of the structures are as shown on the preliminary plans.

2.2	Bridges for this project have a width of structure as follows:

ROADWAY	OUT to OUT

LOCATION	WIDTH	WIDTH

Existing mainline bridges	24 ft.	26 ft.

New mainline bridges	40 ft.	42 ft.

2.3	KDOT standard corral rails, 1’-0” width and 32” height, are provided on all new mainline aa nnn bridges and as the retrofit barrier on all existing bridges unless specified otherwise.  Provisions are made for thrie-beam guard fence connections to the concrete barrier rail at all four bridge corners, for both existing and new bridges.

2.4	Minimum horizontal and vertical clearances are as follows:

2.4.1	Desirable horizontal clearance to obstructions is as required in the KDOT Roadside Design Policy and the "AASHTO Roadside Design Guide" based on design speed, traffic volumes, and slopes.  Site specific exceptions may be necessary to allow for existing conditions.

    - Desirable vertical clearance at grade separations:                                         
 		- 16’-4” for side roads over aa nnn  (Example: US 75)
 		- 15’-4” for local roads under aa nnn
 		- 16’-4” for local roads under aa nnn, if there is an interchange
 		- 15’-4” for state Highways under aa nnn
 		- 16’-4” for state Highways under aa nnn, if there is an interchange

Desirable vertical clearance is a minimum of 23’-0” for railroad overpasses. 

Minimum high water clearance for the aa nnn bridges over aaa Creek is   2’-0”  to the 100 year high water elevation and 3’-0” to the 50 year high water elevation.  Site specific exceptions may be necessary to allow for existing conditions.

    - Clearance requirements for new mainline bridge crossings:

Feature Crossed (Ser. #)	Vertical Clearance	Horizontal Clearance

aaaaaaaaa (nnn)	16’-4”	25’-0”

3.	DESIGN LOADS

3.1	Dead Load

3.1.1	Structural Only  ("A" DL)

(1)	Concrete with reinforcing steel = 	150ft3 (density)

(2)	Structural steel = 	0.2833 in3 (density)

3.1.2	Superimposed Dead Load ("B" DL)

(1)	An allowance of 15ft2 is made for the loading of an initial 1½” thick concrete wearing surface on all transverse slab designs with overlays.

(2)	An allowance of 25ft2 for the loading of a future wearing surface is included in the design of bridge decks with 3” clearance to the top reinforcing.

(3)	Barrier Curbs.

(4)	Allowance for dead loads such as pipes, hand railings, light standards, conduits or other immovable appurtenances is included in the design where appropriate.

3.1.3	Total Dead Load  = "A" DL + "B" DL.

3.2	Live Load

3.2.1	All new bridges are designed for the LRFD HL-93 live load.

3.2.2	The maximum number of design lanes is as specified by AASHTO with allowable reduction for multiple lane loading for substructure design.

3.2.3	Live load plus dynamic allowance deflection does not exceed 1/800 of the span length (no sidewalks).  All beams for prestressed concrete beam or steel girder bridges are considered to act together and have equal deflection.

3.2.4	Dynamic allowance provisions, in accordance with LRFD are applied to superstructure and substructure elements above the footings.  Impact is not applied to substructure units below tops of footings.

3.3	Fatigue

    - Structural steel beams and plate girders are designed for a minimum of LRFD fatigue using the HL-93 (One Lane Loaded) design loading.

3.3.2	Structural steel is designed for applicable AASHTO Fatigue Categories for redundant load path structures.

3.3.3	For weathering steel design, the allowable fatigue stress range for Category A details is reduced to the Category B allowable provided by AASHTO.

3.4	Thermal Forces

3.4.1	The design mean temperature is 60° F.

3.4.2	Forces and moments due to temperature rise and fall are calculated for the following temperature ranges:

(1)	Steel:

Coefficient of Thermal Expansion = 6.5x10-6 / °F

Temperature Range =  -30°F  to  +120°F

Temperature Rise 	60°F

Temperature Fall 	90°F

(2)	Concrete:

Coefficient of Thermal Expansion = 6.0x10-6 / °F

Temperature Range =  0°F  to  +80°F

Temperature Rise 	20°F

Temperature Fall 	60°F

3.5	Longitudinal Forces

3.5.1	Longitudinal forces are computed in accordance with LRFD.

3.5.2	Forces at elastomeric bearings are based on a shear modulus for 60 durometer hardness.

3.6	Earth Forces

3.6.1	Earth load is assumed to be 120ft3.

3.6.2	A minimum equivalent fluid load of 45ft3 is used to calculate horizontal design forces unless the geotechnical investigation report directs otherwise.

3.7	Seismic Forces

New bridges are designed in accordance with the AASHTO Specifications.  Design for Seismic Performance Category A.

3.8	Wind Loads

Wind loads are computed in accordance with AASHTO LRFD Article 3.8 for a 100 mile per hour wind velocity.

3.9  	Loading Combinations

Load combinations are in accordance with AASHTO LRFD 3.4, Table 3.4.1-1.

4.	MATERIALS

4.1	Concrete

Location  	 	   Grade  		f ‘c (ksi)

Barrier Curbs		4.0 (AE)(SA)	  4.0

Slabs and Diaphragms	4.0 (AE)(SA)	  4.0

Prestressed Concrete	Special	  4.5 (min.)

Substructure Above Footings	4.0 (AE)	  4.0

Footings		4.0	  4.0

Drilled Shafts		4.0	4.0

4.2	Reinforcing Steel

4.2.1	Reinforcing steel is ASTM A615 (Grade 60).

4.2.2	All reinforcing steel bends conform to CRSI Standards or as noted otherwise.

4.2.3	All reinforcing steel in the deck slab, barrier curbs, and integral abutments is epoxy coated.

4.2.4	Reinforcing steel in substructure members which are adjacent to traffic has a minimum concrete cover of 3.0”.

4.2.5	The maximum length for reinforcing bars is 40’-0” for #4 bars and 60’ for #5 bars and larger.  Cut reinforcing bars to CRSI tolerances.

4.2.6	No allowance is made in bar length except for corrections associated with standard hooks and special bends.

4.2.7	All bent bar dimensions are out-to-out.

4.3	Prestressing Steel

4.3.1	Prestressing steel is ½” nominal diameter Grade 270 "Uncoated Seven-Wire Low Relaxation Strands for Prestressed Concrete", ASTM Designation A416.  Minimum ultimate strength of strands is 41.3KipStrand.

4.3.2	Initial tensile force applied to each strand is 75 percent of ultimate strength or 31.0KipStrand.

4.4	Structural Steel

4.4.1	Weathering steel (unpainted) is used on all new structural steel bridges.  The KDOT Bridge Policy on Painting of Weathering Steel is to be followed.

4.4.2	Structural steel conforms to the following ASTM (AASHTO) requirements:

(1)	A709 Grade 36			Fy=36 ksi

(M270) Grade 36Tn			Fy=36 ksi

(2)	A709 Grade 50W 			Fy=50 ksi

(M270) Grade 50WTn 		Fy=50 ksi

4.4.3	Modulus of Elasticity =  29 x 106 psi.

4.4.4	Rolled beam sections conform to ASTM A6 requirements.

4.4.5	Reference KDOT Bridge Design Manual, page 16 of Appendix A, for steel plate size availability.  Use 1” increments for girder flange widths.  Use 2” increments for girder web depths.

5.	SUPERSTRUCTURE DESIGN

5.1	Concrete Deck Slabs

5.1.1	The slab is designed using the AASHTO Load & Resistance Factor Design Method for transverse slab designs.

5.1.2	The top ½” of the deck on bridges with overlays, or the top 1” of the deck on bridges without overlays, is neglected as part of the effective depth to account for long-term wear and deterioration under service conditions.

5.1.3	Transverse bars are straight with staggered spacing top and bottom.

5.1.4	Top longitudinal bars are a minimum of #5 bars at 12” centers. Additional bars are added over piers for continuous beam or girder designs.

5.1.5	The top reinforcing steel cover is 3” and the bottom cover is 1½” on all new deck slabs.

5.1.6	Stay-in-place prestressed panels may be used on prestressed concrete beam bridges.  Stay-in-place steel forms or prestressed panels are not used on steel beam or girder bridges.

5.2	Structural Steel

5.2.1	Steel Rolled Beams or Welded Plate Girders

(1)		Design is by the AASHTO LRFD design method for non-hybrid girders.

(2)		Composite design is used.

a.	The effective slab thickness is reduced by ½” for bridges with overlays, or by 1” for bridge decks without overlays, to account for long-term wear.

b.	A value of n = Es/Ec of 8 is used.

(3)		Minimum flange size for the welded plate girders is ¾”  by 10” for spans less than 80’, 1” by 12” for spans greater than 80’ but less than 150’, and 1” by 15” for spans greater than 150’.

(4)		Minimum web thickness for welded plate girders is ⅜”.

(5)		Steel sections for welded plate girders need not be symmetrical in negative moment regions.

(6)	Longitudinal deck slab reinforcing steel shall be considered as part of the composite section in negative moment regions.

(7)		Headed stud anchors are ⅞” diameter.  Stud anchors are placed in both positive and negative moment regions.  The maximum spacing should not exceed 2’ except over interior supports of continuous spans.

(8)		The material for steel members meets the Charpy V-notch test for notch toughness (Zone n) when so designated by the KDOT Bridge Manual.  Members requiring Charpy V-notch testing are designated as such on the plans.

(9)		Welded plate girders are cambered for dead load deflection plus vertical curve correction.

(10)	Top of pavement elevations at centerline girder and dead load deflections are given in the plans at tenth points of spans. Dead load deflections are also given at field splices along with erection elevations at top of top splice plate.

5.2.2	Diaphragms and Cross Frames

Pier and intermediate diaphragms are solid bent plates for depth of beam or girder up to 4’-0” and cross frames for girder depth of 4’-0” or greater.  Spacing between diaphragms shall not exceed 25’-0”.  Additional           diaphragms in outside bays may be required for stability during              construction of bridges with large slab overhangs or shallow girders.

5.2.3	Field Splices

(1)		Field splices are generally located at points of dead load        contra-flexure.

(2)		Field splices are designed using the AASHTO LRFD Method.

(3)		Member weight and length between field splices follow the criteria as outlined on Section 3.3.2.3.5 of the KDOT Metric Bridge Manual.

(4)		The minimum number of bolts on each side of a flange splice plate is 6, placed in 2 rows of 3.

(5)		All bolts for field splices are ASTM A325 high strength bolts (Type 3).  For weathering steel design, an allowable shear stress of 13 ksi (Class a surface, oversized holes) is used for slip-critical connections.

(6)		Oversized holes are allowed in one ply of the secondary connections.  All web/flange joints are made with standard sized holes only.

5.3	Prestressed Concrete

5.3.1	Prestressed Concrete Stresses (Pretensioned, Precast)

(1)		Ultimate Compressive Strength, f 'c = 5 ksi, min. (28 days).

(2)		Compressive strength before releasing of prestressing strands, f 'ci, minimum of 4 ksi.

(3)	Temporary allowable stresses before losses due to creep and shrinkage:

Compression:	fc = 0.60f 'ci

Tension:		fc = 0.25(f 'ci)1/2

(4)		Allowable stresses after losses have occurred:

Compression:	(a)	under all load combinations except as stated in (b) & (c):

fc = 0.60f 'c

(b)	due to effective prestress plus dead load:

fc = 0.40f 'c

(c)	due to live loads plus 0.5(PS+DL):

fc = 0.40f 'c

Tension:	fc = 0 psi	(except as noted below)

fc = 0.25(f 'c)1/2	(including long term creep and shrinkage losses caused by the positive moment connection)

(5)	Prestress losses are calculated in accordance with AASHTO LRFD.

5.3.2	Deflection and camber tables are shown.

6.	SUBSTRUCTURE DESIGN

6.1	Columns

6.1.1	Columns are designed by the AASHTO Load Factor design method.

6.1.2	Round columns are designed as tied columns and reinforced with spiral ties that are ⅜” diameter or ½” diameter smooth bars.  Minimum column diameter is 2’-6”.

6.1.3	Pier cap beams are a minimum of 2” wider than columns on each side.

6.1.4	Column reinforcing steel is spliced as follows:

(1)	No splices shall be used for column heights less than 20’.

(2)	For column heights greater than 20’, splice one-half of the main column bars at the top of footing and one-half of the main column bars at one splice length above the top of footing.

6.2	Footings and Abutments

6.2.1	Footings are designed by the AASHTO Service Load design method.

6.2.2	The tops of footings are placed a minimum of 2’-0” below final ground line.

6.2.3	Pile Embedment:

(1)	Pier Footing  = 1’-0” (minimum)

(2)	Non-integral Abutment Footing = 1.5 x (Pile Diameter)

(2)	Integral Abutments  = 2’-0”

6.2.4	Bottom footing reinforcing steel is placed 3” clear of the bottom of footing.

6.2.5	Pile loads, drilled shaft loads and spread footing reactions are calculated using service loads.

7.	MISCELLANEOUS

7.1	Drainage

7.1.1	Deck drains are provided on all structures where they are required by design.  Refer to the KDOT English Bridge Manual Section III.C for guidelines to be followed.

7.1.2	Deck drains are designed to prevent encroachment of water in the traffic lane.  Design storm frequency and spread limits are as described in the Drainage Design Criteria for this project.

7.1.3	Drains are located such that water will not fall directly onto lower roadways. Drain locations are shown on the Construction Layout.

7.1.4	Abutment Strip Drains will be used as drainage systems behind abutments for new and existing bridges.

7.2	Bearings

7.2.1	Elastomeric Bearing Devices:

(1)	These devices are limited to thermal movement not to exceed 3”.

(2)	Any bearing adjustment required due to profile grade and cross slope is made with beveled sole plates.  Tapered pads are not used.

(3)	The elastomer is neoprene with a durometer of 60.

7.2.2	TFE / Elastomeric Bearing Devices:

(1)	These devices are used at non-integral abutments or piers when the thermal movement exceeds 3”.

(2)	Any bearing adjustment required due to profile grade and cross slope is made with beveled sole plates.  Tapered pads are not used.

(3)	The elastomer is neoprene with a durometer of 70.

7.2.3	Rocker and Pedestal Bearings:

When thermal movement and/or vertical load dictates, steel rocker and pedestal type bearing devices are used.  Refer to the KDOT Bridge Manual Section 3.2.12 for guidelines to be followed.

7.3	Utilities

Allowances are made to provide for utilities as required at each bridge location.

7.4	Lighting

Allowances are made to provide for lighting as required at each specific bridge location.  Electrical conduit not being used on existing bridges can be removed.

7.5	Signing

Allowances are made to provide for signing as required at each bridge location.

7.6	Expansion Joints

Deck expansion joints having total movement of 4” or less are strip seal joints.  (A709 Grade 50W steel is not allowed.)

8.	CONTRACT PAY ITEMS

The following is a list of common pay items and units:

Item Name			Unit

Class I Excavation			Cu. Yds.

Class II Excavation			Cu. Yds.

Class III Excavation			Cu. Yds.

Concrete (Grade 4.0)(AE)		Cu. Yds.

Concrete (Grade 4.0)(AE)(SA)		Cu. Yds

Reinforcing Steel (Grade 60)		Lbs.

Reinforcing Steel (Grade 60)(Epoxy Coated)	Lbs.

Prestressed Concrete Beam (K-__)		Ln. Ft.

Structural Steel (A709) Grade 36		Lbs.

Structural Steel (M270) Grade 36Tn	Lbs.

Structural Steel (A709) Grade 50W		Lbs.

Structural Steel (M270) Grade 50WTn	Lbs.

Headed Stud Anchor			Each

Bearing Device			Lbs.

Elastomeric Bearing Device		Each

TFE / Elastomeric Bearing Device		Each

Steel Pile				Ln. Ft.

Steel Sheet Piling			Ln. Ft.

Predrilled Pile Hole			Ln. Ft.

Drilled Shaft (  _ Dia.)			Ln. Ft.

Permanent Casing			Ln. Ft.

Core Hole (Investigative)		Ln. Ft.

Silica Fume Overlay	(1.5”)		Sq. Yds.

Strip Seal Assembly (Type __)		Ln. Ft.

Abutment Strip Drain			Sq. Yds.

Bridge Backwall Protection System		Sq. Yds.

Protective Coating for Concrete		Sq. Yds.

Substructure Waterproofing Membrane	Sq. Yds.

Slope Protection (Aggregate)		Cu. Yds.

Slope Protection (Riprap Stone)		Cu. Yds.

Bridge Drainage System		Lbs.

Removal of Existing Structure		Lump Sum

Machine Preparation 	(__ mm)		Sq. Yds.

Area Prepared for Patching (Full Depth)	Sq. Yds.

Area Prepared for Patching		Sq. Yds.

Reinforcing Steel (Grade 60) Repair (Set)	Lbs.

Drill and Grout				Each

Hydrodemolition			Sq. Yds.

Painting of Structural Steel		Lump Sum

Abutment Repair			Each

Reset Existing Bearing Devices		Each